The restoration of facial aesthetics and improvement of quality of life are primary goalsfor patients with facial deformities requiring maxillofacial prostheses [1].https://doi.org/10.3390/ma16165580 https://www.mdpi.com/journal/materialsMaterials 2023, 16, 5580 2 of 10a growing interest in improving the mechanical or physical properties of elastomers byincorporating nano-oxide particles, such as ZnO, TiO2, and CeO2, as fillers in silicone-basedelastomers [1,9,10].Thephysical and chemical properties of silicone elastomers, which are commonly used in facialprostheses, are influenced by factors such as crosslinking within their structure and the typeand concentration of fillers in the elastomer network, as well as thermal initiators, otheradditives, polymerization time, and temperature [4-6].By utilizing unpigmented silicone elastomers and considering variousmaterials, this research aims to contribute valuable insights into the field of maxillofacial prosthetics and enhance our understanding of how these treatments can impactcolor stability in different clinical scenarios.In this regard, the evaluation of disinfectants and nano-oxides on unpigmentedsilicone elastomers serves the purpose of achieving better standardization and facilitatingdata interpretation across various elastomer types.However, to achieve better standardization and facilitate data interpretation across various elastomer types, it is essentialto evaluate the effects of disinfectants and nano-oxides on unpigmented silicone elastomersas well.In this regard, the first part of our study aimed to evaluate the antimicrobial properties ofincorporating nano-oxides and different disinfectants in unpigmented maxillofacial siliconeelastomers [18].Cleaningof silicone prostheses can be achieved through various methods, including mechanicalcleaning using manual brushing or hand washing with neutral soap, as well as chemicalcleaning with nontoxic disinfectants [14].Furthermore, when selecting a disinfectant, consideration should be given to itsantimicrobial properties, compatibility, and inertness, in order to minimize any adverseimpact on the physical properties of the material's surface and preserve them to the greatestextent possible.Moreover, mechanical cleaning methods may contribute to thedeterioration of the silicone material and may not effectively eliminate all accumulated bacterial colonies [1,13,15].Differencesobserved in studies investigating the color and mechanical properties of maxillofacialsilicones after disinfection can be attributed to variations in study design and the materialsused.It was reported that the use of a reflection spectrophotometer as amethod has been widely utilized for assessing the color stability of maxillofacial elastomers.On the other hand, colorimeters also provide color measurements in the CIE L*a*b* format,which allows for mathematical analysis and facilitates the comparison of color parametersbetween different objects [11].This investigation helped to establish the efficacy of these treatments inproviding antimicrobial protection to silicone, which is crucial for its clinical application inmaxillofacial prosthetics.The second part focused on examining how the incorporationMaterials 2023, 16, 5580 3 of 10of these disinfectants and nano-oxides influenced the color stability of different types ofmaxillofacial silicone.Studies have reported the colonization of complex microbial biofilms in maxillofacialprostheses.