Monitor StationsTrack GPS satellites as they pass overheadCollect navigation signals, range/carrier measurements, and atmospheric dataFeed observations to the master control stationUtilize sophisticated GPS receiversProvide global coverage via 16 sites: 6 from the Air Force plus 10 from NGA→Master Control Station• Provides command and control of the GPS constellation• Uses global monitor station data to compute the precise locations of the satellites• Generates navigation messages for upload to the satellites• Monitors satellite broadcasts and system integrity to ensure constellation health and accuracy• Performs satellite maintenance and anomaly resolution, including repositioning satellites to maintain optimal constellation• Currently uses separate systems (AEP → & LADO →) to control operational and non-operational satellites• Backed up by a fully operational alternate master control stationGround Antennas• Send commands, navigation data uploads, and processor program loads to the satellites• Collect telemetry• Communicate via S-band and perform S-band ranging to provide anomaly resolution and early orbit support• Consist of 4 dedicated GPS ground antennas plus 7 Air Force Satellite ControlNetwork (AFSCN) remote tracking stations