Bubble cap trays: A bubble cap tray has riser or chimney fitted over each hole, and a cap that covers the riser.Vapour rises through the chimney and is directed downward by the cap, finally discharging through slots in the cap, and finally bubbling through the liquid on the tray.The cap is mounted so that there is a space between riser and cap to allow the passage of vapour.