The authors used an experimental research approach combining mechanical testing, monitoring techniques, and microstructural analysis to study damage and fracture behavior of steel fiber-reinforced concrete (SFRC).Specifically, they employed the following methods and experiments:	o	Three-point bending fracture tests on notched SFRC beams with different steel fiber contents to evaluate bending damage, load-CMOD behavior, flexural-tensile strength, and fracture toughness.o	Scanning Electron Microscopy (SEM) to analyze the microstructure and fracture surfaces after testing, identifying mechanisms such as fiber bridging, debonding, and pull-out.