AbstractThe Industrial Revolution can be termed as the transformation of traditional industrial practices into new techniquesdominated by the technologies available at that time.The definition of "sustainability trilemma" anew term coined by the authors, and the reasoning for calling the next industrial revolution "Industry 4.0S" (another newterm) rather than Industry 5.0 are also presented. Keywords Industrial revolution . Sustainability . Artificial Intelligence . Internet of things . Collaborative robots . Digital twins . Edge computing . Block chain . Cyber-Physical SystemsReceived: 18 February 2022 / Accepted: 20 January 2023 / Published online: 5 February 2023(C) The Author(s), under exclusive licence to Springer-Verlag France SAS, part of Springer Nature 2023Industry 5.0 or industry 4.0S? Introduction to industry 4.0 and a peekinto the prospective industry 5.0 technologiesAbirami Raja Santhi1,2  . Padmakumar Muthuswamy1,21 3International Journal on Interactive Design and Manufacturing (IJIDeM) (2023) 17:947-979by various electronic devices. Digital sensors and computers became a part of the shop floor. Although the working conditions improved tremendously during this period,exploitation of labor continued, cities became overcrowded,and widespread pollution and environmental degradationbecame common across the world. The ongoing fourth industrial revolution which is commonly referred to as Industry4.0 or 4IR is built upon the third industrial revolution whichrelied on transistors, sensors, and micro-electronics to generate data. The term Industry 4.0 was coined by GermanProfessor Wolfgang Wahlster, in the year 2011 at the Hannover Fair. It is the digital transformation of manufacturingindustries that focuses on automation, interconnectivity, andreal-time process optimization using enabling digital technologies such as Internet of Things (IoT), Machine Learning(ML), Artificial Intelligence (AI), Cyber-Physical Systems(CPS), Cloud computing, Additive Manufacturing (AM),Digital twins, Cybersecurity and so on to communicate andcontrol each other [4, 5]. In other words, it can be calledthe computerization of manufacturing in which advanceddigital technologies are married to industrial machines andprocesses. The interconnection of these technologies intothe manufacturing setup is to achieve operational efficiency,productivity, and automation to the highest possible extent[6]. This in turn creates a manufacturing ecosystem that issmart, connected, and driven by data. The fundamental model of Industry 4.0 can be dividedinto digital or computing technologies that are marriedwith the systems of the physical world. While AI, ML, BigData, Cloud Computing, and cyber security form a part ofthe core computing technologies, other technologies suchas Automation and Robotics, IoT, CPS, and AM form thephysical part. These technologies together realize the benefits of Industry 4.0 systems to enable agile, flexible, andon-demand manufacturing which is an essential part ofsmart manufacturing or smart factories. While there aretremendous advantages for the industries in implementingthese technologies to achieve competitive advantage andhigher operational efficiency, there is widespread apprehension about the potential job loss for low-skilled laborers dueto the high level of automation which can cause economicimbalance and greater inequality in the society [7]. The various stages of the Industrial Revolution with their timeline,driving force, and technologies are shown in Fig. 1. While the world is still trying to adapt and realize thepotential of Industry 4.0, some industrialists and scholarshave started envisioning and discussing the next IndustrialRevolution, Industry 5.0. If Industry 4.0 is about digitallyconnecting machines to enable a seamless flow of data andthe highest possible optimization, Industry 5.0 is believedto bring humans back into the game for collaboration andintroduce the human touch to manufactured products whilesimultaneously focusing on sustainable manufacturing [8,9]. Elon Musk, a visionary entrepreneur and CEO of oneof the most highly automated factories in the world, TeslaInc., has acknowledged the downside of excessive automation through his tweet in April 2018, "Yes, excessive automation at Tesla was a mistake.Humans are underrated". He went on to admit that robotshave slowed down production, and humans, not machines,were the solution. This is in line with the predictions that thenext big thing would be the collaboration between humans,robots, and digital technologies. 1.1 Need for the studyDespite the ongoing adaptation of Industry 4.0 in various sectors and growing discussions on Industry 5.0, thispaper aims to provide a brief background on the enablingtechnologies of Industry 4.0 and their application in various functions of manufacturing industries, the prospectiveIndustry 5.0 technologies and their potential applicationsFig. 1 The various stages of the Industrial Revolution1 3948International Journal on Interactive Design and Manufacturing (IJIDeM) (2023) 17:947-979and attempts to answer the following five research questions (RQ),RQ1. What are the enabling technologies of Industry 4.0and their application in manufacturing industries? RQ2. What are the socio-economic challenges of Industry 4.0 technologies? Why must the industries overlookthese technologies and upgrade to the prospective Industry5.0 technologies? RQ3. What are the prospective technologies of Industry5.0 and their application in manufacturing industries? RQ4. What is "sustainability trilemma" and how doesIndustry 5.0 technologies help to overcome it?Why "Industry 5.0" must be called "Industry4.0S"? The above questions are answered through various sections of this article. The Sect. 2 introduces the variousenabling technologies of Industry 4.0 and their applicationin manufacturing industries which aims to answer RQ1. Thesocio-economic challenges of Industry 4.0 technologies andthe reason to overlook these technologies and upgrade tothe prospective Industry 5.0 technologies are explained inSect. 3 which answers RQ2. The Sect. 4 of the article discusses the predictions from industry leaders and providesthe definitions of Industry 5.0 as quoted by industries andscholars. The scholarly articles related to the prospectiveIndustry 5.0 technologies and their application in manufacturing industries are discussed in Sect. 5 which answersRQ3. The definition of "sustainability trilemma" a newterm coined by the authors, the sustainability dimensionof Industry 5.0 technologies, and how the industries canembrace sustainable development using the technologiesare discussed in Sect.1.2 Methodology and structureTo understand the enabling technologies of Industry 4.0,prospective Industry 5.0 technologies, and their respectiveapplication in various functions of manufacturing industries, a comprehensive literature review was performed.The first three industrial revolutions were driven respectively bymechanization, electrification, and automation which had gradually transformed the agrarian economy into a manufacturing-based economy.To be precise, my mistake.RQ5.6 which answers RQ4.