The importance of performance evaluation of mobile networks is inspired by the need toutilize the limited radio resource in the most efficient manner.This study forms a basis for the regular inspection and optimization of mobile networks in theUniversity of Ilorin, as well as creates a guide for path loss modeling suitable for the campusterrain.The authors in [19], conducted a pilot studyto benchmark and evaluate the QoS experienced by mobile wireless users in Pakistan bydeploying an android application (My Speed Test PK) for mobile customers to measure theperformance of the five broadband operators (Ufone, Telenor, Zong, Warid, and Mobilink) inPakistan.While the operators focus on metrics like the bit error rate (BER), frame erasure rate (FER),and mean opinion score (MOS), the Nigerian Communications Commission (NCC) hasidentified some measurable key performance indicators (KPIs) to evaluate the success ofnetwork operators in Nigeria vis-a-vis subscribers' perception.As a national regulatory body vested to establish minimum QoS standards in service deliveryfor mobile network operators (MNOs) and internet service providers (ISPs), the NCC requiresthat each network provider reports quarterly, on customer satisfaction in (rural, urban and suburban) Nigeria [13].Results from the study emphasized the need for optimalcoverage on all the mobile networks, as well as to have the radio frequency (RF) air interfaceparameters configured to avoid problems of ping-pong handovers, and poor power control,which causes interference and low RSSI among mobile stations (MSs).Arelated study in [5] employed the drive test method with statistical analysis models (chi-squareand Fisher's tests) for the performance benchmarking of GSM and Universal MobileTelecommunications System (UMTS) network operators in Ankara, Turkey.The continuous increase indemand, for high-quality service, coupled with the future telecommunication network target ofproviding integrated services with multimedia applications, over low-powered mobilecomputing devices, has further increased the need for evaluating the performance of mobilenetworks [1 - 4]. This justifies previous statistics by the NCC in May 2016 toApril 2017, that amongst the four major MNOs, only one performed outstandingly (satisfyingthe minimum requirement and beyond) in network accessibility, while one of the MNOsperformed poorly in network retainability [14].From the drive test analysis, the problem identified acrossthe three GSM clusters were - reduced power output, poor transmission line, increasedinterference, coverage hole, cross feeder, overshooting, amongst others.However, fromtechnical and regulatory perspectives, network performance evaluation is necessary for assessingand measuring the coverage, capacity, and quality of mobile networks with respect to the servicelevel agreement (SLA) signed with the regulators.While past studies have critically evaluated the performance of GSM and WCDMA/UMTSnetworks from customers' viewpoint, the study in [25] demystifies the root cause of poornetwork signal strength and its impact on the QoS provisioning by MNOs.In a populated urban area with a literate population such as the University of Ilorin, radioresource availability and reliability is a factor that determines the success of mobilecommunication.This incessant complaint which has made many staff andstudents, each, to subscribe to more than one network operators' service, is an indicator of poorQoS in the University of Ilorin, and by extension most urban centers in Nigeria.The analysis of the performance obtained by mobile wirel meticulous measurement for the mobile network operators (EE, O2, Vodafone, and Three) in theUnited Kingdom [23].It has, increasingly, become evident and pronounced that the Nigeriantelecommunication sector has been plagued with problems, causing frustration amongconsumers.