Abstract The presence of organic compounds in drinkingwater is well recognized in many developing countries;however, the occurrence of organic contaminantsin the groundwater of Saudi Arabia, which is the mainsource of drinking water in the country, is not welldocumented.Themost relevant compounds in the 71 water sampleswith respect to highest median and maximum concentrationswere 1H-benzotriazole, tris(1-chloro-2-propyl)phosphate (TCPP), methylbenzotriazoles,carbamazepine and its metabolite, DEET, sulfamethoxazole,tris(isobutyl)phosphate (TiBP), tris(2-chloroethyl)phosphate (TCEP), perfluorooctanoic acid(PFOA), perfluorooctanesulfonic acid (PFOS), anddiclofenac.Trihalomethanes(THMs) were the most frequently detectedVOC groups; however, a few compounds, such as methyltert-butyl ether (MTBE), ethylene dibromide (EDB),and dibromochloropropane (DBCP), had regional orlocal occurrence patterns.The microorganicssuch as caffeine, N,N-diethyl-m-toluamide(DEET), bisphenol A, anti-microbial agents, pharmaceuticals,and contaminants including chlorinated solventsand THMs, petroleum hydrocarbons, pesticides,and other industrial compounds were detected in thegroundwater.Lapworth et al. (2015) presented the results froma reconnaissance survey of micro-organics (MOs) inChalk groundwater, which included pharmaceuticals,personal care products, and pesticides and theirtransformation products across the major Chalkaquifers of England and France.These compounds werefound in low concentrations, though some of the compoundslike imidacloprid, methiocarb, diclofenac,macrolide antibiotics, ciprofloxacin, EHMC, andBHTwere detected in concentrations in excess of 100ng/L.Manamsa et al. (2016) reportedthat different types of micro-organic (MO) compoundsused for manufacturing, food production, andhealthcare were frequently detected in the aquaticenvironment.Kerndorff et al. (1992) reported that approximately1200 organic contaminants have been identified inwaste disposal site groundwater in Germany and theUSA.Among the various chemical substances present inwater, organic compounds were detected to varyingdegrees in some regions of the world to determine theirimpact on human health (Friesel 1987; Kerndorff et al.1992).Twenty domestic drinking water wells were tested bySchaider et al. (2016) in a sand and gravel aquifer inMassachusetts, USA, for 117 organic wastewatercompounds.A national comprehensive study was carriedout to assess the occurrence of organic compoundsin groundwater wells used for drinking water purpose,in different regions of Saudi Arabia.Increasinglevels of pollution and salinity of groundwater resourcesis increasingly affecting the ability to usethe scarce water resources available (UN-ESCWAand BGR 2013).In general, the concentrations of these compoundsin the samples were below the environmentalquality standard (EQS) threshold values except the concentrationof PFOS, which exceeded the annual averagewater EQS limit of 0.65 ng/L along the whole river.Most of the wells containing organiccompounds were located in residential, industrial, andagriculture areas.A recent boom in the industrialsector has led to the production of different typesof inorganic and organic chemicals resulting indrainage disposal of chemical wastes.The presenceof chemicals and other organic pollutants in wastewaterscan pollute the groundwater through deeppercolation (Solomons and Fryhle 2004).They detected 27 organic wastewatercompounds which included 12 pharmaceutical andpolyfluoroalkyl compounds, and four organophosphateflame retardants.It can be said that the problem of groundwater contaminationwith organic compounds in Saudi Arabia isnot acute and is manageable at present.The widespread occurrenceof VOCs indicates the vulnerability of many of the USaquifers to low-level VOC contamination.Keywords Groundwater .Organic compounds .Groundwater contamination .