The conversion of energy from one form to anotheroften affects the environment and the air we breathin many ways. Each liter of gasoline burned by a vehicleproduces about 2.5 kg of CO2An average car driven about 13,500 miles a year andconsumes about 600 US gallons (1680 kg; 1 US gallon is3.78541178 litres while an imperial gallon is 4.546092 litres)of gasoline emits about 12,000 lbm of CO2to the atmospherea year, which is about four times the weight of a typical car.Global climate change is due to the excessive use of fossil fuels(e.g., coal, petroleum products, and natural gas in electric powergeneration, transportation, buildings, and manufacturing), and ithas been a concern in recent decades. Under normal conditions,vegetation consumes CO2 and releases O2 duringphotosynthesis, thus keeps the CO2 concentration in theatmosphere in check. A mature, growing tree consumes about12 kg of CO2 a year and exhales enough oxygen to support afamily of four. However, deforestation and the huge increase inCO2 production in recent decades has disturbed this balance. In a 1995 report, the earth world's leading climate scientistsconcluded that the earth has already warmed about 0.5?Cduring the last century, and they estimate that the earth'stemperature will rise another 2?C by the year 2100. A rise of this magnitude can cause severe changes in weatherpatterns with storms and heavy rains and flooding at some partsand drought in others, major floods due to the melting of ice atthe poles, loss of wetlands and coastal areas due to rising sealevels, variations in water supply, changes in the ecosystem dueto the inability of some animal and plant species to adjust to thechanges, increases in epidemic diseases due to the warmertemperatures, and adverse side effects on human health andsocioeconomic conditions in some areas. The seriousness of these threats has moved the United Nationsto establish a committee on climate change. A world summit in1992 in Rio de Janeiro, Brazil, attracted world attention to theproblem. 162 nations signed an agreement prepared by thecommittee in 1992 to control greenhouse gas emissions. In December 2011, countries agreed in Durban, SouthAfrica, to forge a new deal forcing the biggest pollutingcountries to limit greenhouse gas emissions. The KyotoProtocol was extended to allow five more years to finalize awider agreement. The goal was complete producing a new,legally binding accord to cut greenhouse gas emissions by2015 that would come into force by 2020. The conference includedparticipants from 196 nations. The main result of the conference was the establishment of agoal to limit global warming to less than 2?C compared topreindustrial times. According to the agreement, human-madegreenhouse emissions should be eliminated during the secondhalf of the 21st century. Greenhouse gas emissions can be reduced by increasingconservation efforts, improving conversion efficiencies, and meetnew energy demands by using renewable energy (e.g.,hydroelectric, solar, wind, and geothermal energy) rather thanfossil fuels.Smog is made-up mostly ofground-level ozone O3, but also contains numerous otherchemicals including carbon monoxide CO, particulate mattersuch as soot and dust, volatile organic compounds VOCssuch as benzene, butane, and other hydrocarbonsUnlike the useful ozone layer high in thestratosphere that protects the earth from the sun'sharmful ultraviolet rays, ozone at ground level is apollutant with several adverse health effects.A significantportion of the VOC or HC emissions is caused by theevaporation of fuels during refuelling or spillage during spitback or evaporation from gas tanks with faulty caps that donot close tightlyOzone and SmogSmog is the dark yellow or brown haze that builds up in alarge, stagnant air mass and hangs over the populated areason calm, hot summer days. Ground-level smog and ozone formin urban areas with heavy traffic or industry, but theprevailing winds can transport them several hundredmiles to other cities (i.e., pollution knows noboundaries, it is a global problem)Ozone irritates eyes and damages the air sacs in the lungswhere oxygen and carbon dioxide are exchanged, causingeventual hardening of this soft and spongy tissue. Smog also contains suspended particulate matter (e.g., dustand soot emitted by vehicles and industrial facilities), whichirritate the eyes and the lungs since they may carry acids andmetals compounds such as acids and metalsAcid RainFossil fuels are mixtures of various chemicals, includingsmall amounts of sulfur.The Greenhouse Effect: Global Warming and ClimateChangeGlass at thicknesses encountered in practice transmits over90 percent of radiation in the visible range and is practicallyopaque (non-transparent) to radiation in the longerwavelength infrared regions (i.e., allows the solar radiation toenter freely but blocks the infrared radiation emitted by theinterior surfaces). The surface of the earth, which warms up during the dayas it absorbs solar energy, cools down at night by radiating partof its energy into deep space as infrared radiationCarbon dioxide (CO2), water vapour, and traceamounts of some other gases (e.g., methaneand nitrogen oxides) act like a blanket andkeep the earth warm at night by blocking theheat radiated from the earth, Fig. Many people die each year due to heart and lungdiseases related to air pollutionHundreds of elements and compounds (e.g.,benzene and formaldehyde are emitted during thecombustion of coal, oil, natural gas, and wood inelectric power plants, vehicles engines, furnaces,and even fireplaces).Some compounds are addedto liquid fuels for various reasons (e.g., Methyltertiary-butyl ether (MTBE) is added to raise theoctane number of the fuel and to oxygenate thefuel in winter months to reduce urban smog).Thelargest source of air pollution is motor vehiclesThe pollutants released by vehicles are usually grouped ashydrocarbons (HC), nitrogen oxides (NOx), and carbonmonoxide (CO).Volcanic eruptions and hot springs alsorelease sulphur oxides (the cause of the rotten egg smell)The sulphur oxides and nitric oxides react withwater vapour and other chemicals high in theatmosphere in the presence of sunlight to formsulfuric and nitric acids (Fig.2.62).2.65).2.66).